Private Investment and Digital Entrepreneurship (PRIDE) Project

Bangladesh Economic Zones Authority (BEZA)

Environmental and Social Assessment (ESA)

Package No. BEZA S-135

Final Report

Appendices to ESA Report Vol II

January 2020

Appendix D: Assessment of Air Quality and Water Quality

## Air Quality

### Introduction

The BSMSN area will generate huge traffic after full development both for passenger and goods. There will be wide range of vehicles such as two and three axle/ multi axle trucks, bus, car, two-wheeler etc. These type of transportation system are major contributors to air pollution as these emit harmful pollutants which has adverse impact on environment. It is difficult to project air pollution as because pollutant enters the air by different amounts at different times and also varies with places. It is expected that air pollutants which might be discharged by the operations of tenant in the industrial and logistic area would be well controlled and managed by the tenants. For indicative assessment of air quality of the project area, projection has been done for some pollutants due to vehicle emission only.

### Forecast Area

The areas examined to forecast is the project area (Zone 2A and 2B of BSMSN) shown in the Figure 1-1. The project area is about 1450 acres. At present, it is observed that the concentration of some pollutants such as SO2, NOX, CO, PM2.5, PM10 and SPM are below the allowable limit as per DoE standard (Bangladesh). All the surveyed data are given in the baseline data collection section of main report.

### Forecast Period

Air quality will be forecasted for the operation phase when all the tenants in the project area will start their operation. It is expected that by the year 2040, the BSMSN area will be fully developed and operative. There are different phases of this project where air quality may be deteriorated significantly due to construction issues and other factors. Impact of air pollution is expected to be limited because construction vehicles will be little by the year 2040.

### Forecast Method

The air quality due to vehicle emissions have been calculated is based on the rapid assessment manual of WHO (1993). Five (05) tracing substance such as SPM, PM10, PM2.5, SOX, NOX and CO have been measured at baseline condition. Five (05) tracing substance such as Dust, SOX, NOX, CO and VOC have been predicted based on the guidelines of WHO.

The emission factor of two-wheeler, car, microbus, mass transport-bus and tucks is shown in the Table 1-1.

Table 1-1: Emission Factor for Different Types of Vehicle

| SL | Parameter | Emission factor (g/km) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Two-Wheeler | Car | Micro Bus/CNG | Mass Transport-Bus | Heavy Truck (Gasoline) | Heavy Truck (Diesel) | Medium Truck | Light Commercial Vehicle (LPG) |
| 1 | Dust | 0 | 0.05 | 0.05 | 0.9 | 0.6 | 1.3 | 0.9 | 0 |
| 2 | SO2 | 0.77 | 1.725 | 1.725 | 6.11 | 3.35 | 6.1 | 4.15 | 0 |
| 3 | NO2 | 0.3 | 1.83 | 1.83 | 13.9 | 7.5 | 19.8 | 14.4 | 2.75 |
| 4 | CO | 20 | 1.29 | 1.29 | 2.1 | 50 | 3.1 | 2.9 | 1.15 |
| 5 | VOC | 3 | 0.17 | 0.17 | 1.7 | 3.5 | 2.4 | 0.8 | 1.03 |

Source: WHO (1993)

### Freight and Passenger Traffic

The freight volume has been estimated by multiplying project area, industrial land use factor and number of truck trips per hectare. It is found from BSMSN master plan that the industrial land use is taken about 54% of the total project area. It is difficult to predict traffic production and attraction from an economic zone. According to BSMSN master plan, the number of truck trips per hectare would be around 8 trucks by the year 2040. Therefore, the freight (truck trip nos.) value is 2536 by the year 2040. The freight no. is sub-divided into heavy truck trip, medium truck trip and light truck trip. This is shown in Table 1-2.

Table 1-2: Truck Trip Number by the Year 2040

| Truck Type | Freight (Total Truck Trip Number) | Percentage (%) | Trip No. |
| --- | --- | --- | --- |
| Heavy Truck | 2536 | 40 | 1014 |
| Medium Truck |  | 30 | 761 |
| Light Commercial Vehicle |  | 30 | 761 |

The trips nos. of two-wheeler, car, IPT (microbus/ CNG) and mass transport (bus) for 2040 is shown in Table 1-3.

Table 1-3: Passenger Traffic Trip Number by the Year 2040

| Vehicle Type | Trip Number |
| --- | --- |
| Two-wheeler | 489 |
| Car | 326 |
| IPT (microbus/ CNG) | 1222 |
| Mass transport (bus) | 1222 |

Source: BEZA (2019b)

### Trip Distribution

All the trips generated by trucks, two-wheeler, car, IPT (microbus/ CNG) and mass transport (bus) are considered to be through two roads i.e. Access Road 1 (Sheikh Hasina Avenue) and Access Road 2 (through Zoararganj and Takerhat). The alignment of this road is shown in Figure 1-1.

Figure 1-1: Access Road Alignment

The length of the Access Road 1 (Sheikh Hasina Avenue) & Access Road 2 (through Zoararganj and Takerhat) are about 19.9 and 26.5 km respectively. It is assumed that 80% of vehicle will move through Access Road 1 (Sheikh Hasina Avenue) & 20% of vehicle will move through Access Road 2 (through Zoararganj and Takerhat) as access road is the main access to BSMSN area.

The nos. of trips through Access Road 1 (Sheikh Hasina Avenue) and Access Road 2 (through Zoararganj and Takerhat) are shown in Table 1-4.

Table 1-4: Trip No. through Access route 1 & Access route 2 by the Year 2040

| Types | Trip No. |  |
| --- | --- | --- |
|  | Access route 1 (Sheikh Hasina Avenue) | Access route 2 (Zorarganj-Takerhat) |
| Two-Wheeler | 391 | 98 |
| Car | 261 | 65 |
| Micro Bus/CNG | 978 | 244 |
| Mass Transport-Bus | 978 | 244 |
| Heavy Truck | 39 | 10 |
| Medium Truck | 29 | 7 |
| Light Commercial Vehicle | 29 | 7 |

### Forecast Result

The pollution loading has been calculated by multiplying nos. of trips made by each vehicle type, length of access route and emission factor for each vehicle types. The pollution loading is shown in Table 1-5 and Table1-6.

Table 1-5: Pollution Loading through Access route 1 by the Year 2040

| Sl. | Parameter | Pollution loading (Kg/day) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Two-Wheeler | Car | Micro Bus/CNG | Mass Transport-Bus | Heavy Truck (Gasoline) | Heavy Truck (Diesel) | Medium Truck | Light Commercial Vehicle (LPG) | Total |
| 1 | Dust | 0 | 6 | 23 | 420 | 2 | 19 | 13 | 0 | 484 |
| 2 | SO2 | 143 | 215 | 806 | 2854 | 13 | 91 | 58 | 0 | 4180 |
| 3 | NO2 | 56 | 228 | 855 | 6492 | 28 | 297 | 202 | 39 | 8197 |
| 4 | CO | 3737 | 161 | 603 | 981 | 187 | 46 | 41 | 16 | 5771 |
| 5 | VOC | 560 | 21 | 79 | 794 | 13 | 36 | 11 | 14 | 1530 |

Table 1-6: Pollution Loading through Access route 2 by the Year 2040

| Sl. | Parameter | Pollution loading (kg/day) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Two-Wheeler | Car | Micro Bus/CNG | Mass Transport-Bus | Heavy Truck (Gasoline) | Heavy Truck (Diesel) | Medium Truck | Light Commercial Vehicle (LPG) | Total |
| 1 | Dust | 0 | 2 | 8 | 140 | 1 | 6 | 4 | 0 | 161 |
| 2 | SO2 | 48 | 72 | 268 | 950 | 4 | 30 | 19 | 0 | 1391 |
| 3 | NO2 | 19 | 76 | 285 | 2161 | 9 | 99 | 67 | 13 | 2729 |
| 4 | CO | 1244 | 53 | 201 | 327 | 62 | 15 | 14 | 5 | 1921 |
| 5 | VOC | 187 | 7 | 26 | 264 | 4 | 12 | 4 | 5 | 509 |

### Forecasting Pollutants Dispersion

The Air Resources Laboratory’s (ARL’s) HYbrid Single-Particle Lagrangian Integrated Trajectory (HYSPLIT) model is used extensively for atmospheric transport and dispersion of pollutants and various materials. This tool explains how, when, and where potentially harmful materials are atmospherically transported, dispersed, and deposited (Stein et al., 2015). The interactive web through the ARL READY system, this model can be run. This web version has some limitations due to computational complicity (Rolph et al., 2017).

In this model, location of point source, time and pollutant loading has to be provided as input data. In this model pollution loading is given as a point source though in practical it has a spatial and temporal distribution. There are other assumptions (Rolph et al., 2017) to produce this dispersion map which needs to be considered carefully during the modelling. The following dispersion maps (Figure 1-2 to Figure 1-5) show an indicative idea of dispersion of pollutants.

Figure 1-2: Dust Dispersion in Dry Season

Figure 1-3: Dust Dispersion in Wet Season

Figure 1-4: CO Dispersion in Dry Season

Figure 1-5: CO Dispersion in Wet Season

## Water Quality

### Introduction

The storm water, domestic wastewater, and industrial wastewater will fall into Bay of Bengal through Ichakhali khal. Projection of water quality due to water discharge into the receiving water body (Ichakhali Khal) has been done.

### Forecast Area

The areas examined to forecast is the project area (Zone 2A and 2B of BSMSN). The project area is about 1450 acres.

### Forecast Period

Water quality will be forecasted for the operation phase (2040) when all the tenants in the project area will start their operation.

### Forecast Method

The quality and quantity of industrial wastewater discharged from the industrial area during the
operation phase was estimated based on:

- estimated water supply capacity for the industrial area

- estimated quality of treated effluent from the industrial area

The environmental impact of the industrial wastewater on the Ichakhali khal was predicted by using a complete mixing model, as shown in the following formula:

C = (C1 X Q1) + (C2 X Q2)C1+C2

where, C (mg/L): Concentration of pollutant in the Ichakhali khal after complete mixing with wastewater

C1 (mg/L): Concentration of pollutant in the Ichakhali khal at the forecast point before mixing

C2 (mg/L): Concentration of pollutant in the wastewater before mixing

Q1 (m3/s): Flow rate of the Ichakhali khal at the forecast point before mixing

Q2 (m3/s): Flow rate of wastewater discharge before mixing

### Wastewater Generation

As mentioned above, the project area has an area of 1450 acres. Water supply requirement for the project area is about 100 MLD.

Wastewater amount was estimated based on the water supply volume. Wastewater yield ratio to water supply was assumed as 80%. Therefore, maximum wastewater amount generated from the industrial area would be about 80 MLD.

The tentative target value of effluent water quality discharged to the surrounding water bodies is set to 40 mg/l (DoE requirement).

### Development of Hydrological and Hydraulic Model

Mathematical model is a tool, which is widely used for planning and design of project infrastructures considering the interactive responses of other project components or other projects. Mathematical model available at IWM is based on tools developed by DHI, and is under continuous up-gradation as well as verification. There are a number of modules included in DHI tool, among which two modules has been used in carrying out modelling (BEZA,2019a) which include: Rainfall-Runoff (MIKE11 RR) for hydrological modelling and one dimensional hydrodynamic (MIKE 11 HD) for hydraulic modelling.

The advantages of MIKE11 modelling systems are:

- Accurate hydraulic description in rivers/channels which behave pre-dominantly one-dimensional.

- It requires less computational time i.e. less CPU time

- East to overview and comprehend results

- Can modelled many different and complicated structures

- Other problems such as water quality, sedimentation, flood forecasting can be addressed with the same model.

#### Development of Hydrological Model

In MIKE 11 software package, NAM module has been used to develop the hydrological model. The MIKE11 NAM is a lumped conceptual model which simulates the overland, interflow, and base flow components as a function of the moisture contents in four storages. The NAM module can either be applied independently or used to represent one or more contributing catchments that generate lateral inflows to a river network. In this manner it is possible to treat a single catchment or a large river basin containing numerous catchments and a complex network of rivers and channels within the same modelling framework.

The existing South East Regional Model (SERM) available in IWM has been truncated and used for developing BSMSN model for simulating the main river system: Feni River, Muhuri River, Selonia River and Little Feni River. The catchments used to simulate the rainfall-runoff of the catchments for the main rivers are stated in Table 2-1.

Table 2-1: SERM Catchments Used to Develop BSMSN Model

| Name of the catchment | Area (km2) | Name of the catchment | Area (km2) |
| --- | --- | --- | --- |
| SE-05 | 1 | SE-29 | 404 |
| SE-06 | 186 | SE-30 | 301 |
| SE-12 | 88.32 | SE-31 | 607 |
| SE-15 | 150 | SE-32 | 645 |
| SE-16 | 131 | SE-36 | 124.59 |
| SE-27 | 131 |  |  |

For detail modelling of the khal system in and around the BSMSN study area, four major catchments have been identified and delineated based on DEM. The four catchment are:

- Ichakhali khal catchment,

- Bamonsundor khal catchment,

- Saherkhali khal catchment, and

- Domkhali khal catchment.

The catchments areas are generated from the contribution of the major khal along with its tributaries. These major catchments further divided into sub- catchments to assess the run-off pattern of the khals.

The Hydrological Model has been developed using the rainfall and evapotranspiration data collected from BWDB.

The generated catchments have been characterized and classified according to the surface terrain, land cover dynamics (slope, vegetation, obstruction etc.) and run-off flow pattern.

Ichakhali khal catchment

The catchment area of Ichakhali khal catchment is about 86 km2. The Ichakhali khal receives water from Feni reservoir site for irrigation purpose. Barmai Chhara and its distribtaries convey water to Ichakhali khal extending to eastern hills. There are some other local khals which contribute to Ichakhali khal and they all together make the Ichakhali system. It is divided into 10 sub-catchments which are Barmi, Dakhin Tajpur, Dawkhali, Ichakhali, Ichakhali dn, Ichamoti, Irarchara, Kata, Mn Drainage-1 and West Ichakhali. The elevation of this catchment varies from 2.7-158 mPWD.

Bamonsundor khal catchment

The catchment area of Bamonsundor khal catchment is about 129 km2. Mohamaya Khal, Mithanala Rajapur khal and Khontakata khal are the major tributaries of Bamon Sundar khal and make the Bamon Sundar khal system. These tributaries and some other local minor khals convey water from eastern hills to Bamon Sundar khal. It is divided into 10 sub-catchments which are Bamon Sundor, Daborkhali Reg Catch, Durgapur kata chhara, Gopinathpur, Khontakata, Mithanala Rajapur, Mohamaya Khal, Mohamaya Reservoir, Omar Ali and Roypur-Gopalpur. The elevation of this catchment varies from 1.2-258 mPWD.

Hydrological Model Calibration

Parameters of NAM have been calibrated against the observed discharge data near the outlet of the catchments. Discharge data have been collected from Parsuram on the Muhuri River and Kaliarchari on the Feni River respectively. Simulated runoff data show good match with the observe discharge data mainly in dry season (Figure 2.1 and Figure 2.2). Observed monsoon pick data is not available for both stations so pick runoff can’t be compared but the available monsoon discharge data also show good match with the simulated runoff.

Figure 2.1: Calibration plot against discharge at Parsuram on the Muhuri River

Figure 2.2: Calibration plot of against discharge at Kaliarchari on the Feni River

#### Development of Hydraulic Model

In MIKE 11 software package, hydrodynamic module (HD) has been used to develop the hydraulic model. The MIKE 11 HD module uses an implicit, finite difference scheme for the computation of unsteady flows in rivers and estuaries. The module can describe sub-critical as well as supercritical flow conditions through a numerical scheme which adapts according to the local flow conditions (in time and space). The output of MIKE11 NAM model has been used as input of MIKE11 HD model. The internal khal network has been incorporated with the truncate South-east Region Model (SERM) to develop BSMSN HD model after field data collection and processing. The river/khal network of the BSMSN model is given in Figure 2.3.

Figure 2.3: River network for BSMSN model

Hydraulic Model Calibration

The Hydraulic model has been calibrated based on observed water level. The calibration has been done at Dhumghat on Feni River and u/s of Feni Reservoir. The following Figure 2.4 shows water level calibration plot at u/s of Feni Reservoir.

Figure 2.4: Water level calibration plot u/s of Feni Reservoir

### Dependable Flow for Ichakhali Khal

Monthly and seasonal dependable flow based on model simulation data of Ichakhali khal is given in Table 2-2. It is observed that in wet season 80% & 50% dependable flow is about 0.7 m3/s and 2.2 m3/s respectively while in dry season 80% & 50% dependable flow is only about 0.7 m3/s and 11.9 m3/s respectively in Ichakhali khal. It is observed that is dry season the 50% dependable flow is more than monsoon flow. This is because, in dry season the gates of Muhuri Regulator become closed and Ichakhali khal get water from this river. But in monsoon the gates are opened and the khal get very little water from Feni River.

Table 2-2: Dependable flow for Ichakhali Khal

| Period | Dependable flow (m3/s) |  |
| --- | --- | --- |
|  | 80% | 50% |
| Dry (Dec-Feb) | 0.7 | 11.9 |
| Wet (Jun-Oct) | 0.7 | 2.2 |
| Jan | 12.0 | 19.8 |
| Feb | 7.2 | 14.2 |
| Mar | 4.9 | 10.5 |
| Apr | 5.1 | 11.5 |
| May | 0.1 | 0.3 |
| Jun | 0.4 | 2.5 |
| Jul | 1.1 | 3.8 |
| Aug | 0.0 | 17.7 |
| Sep | 0.9 | 2.1 |
| Oct | 0.5 | 1.3 |
| Nov | 0.2 | 0.5 |
| Dec | 3.3 | 13.5 |

Source: BEZA (2019a)

### Present Pollutant Concentration

BOD5 was selected as the representative indicators for evaluating the impact on the environment since this parameter are the most basic parameters of concern for the environment as well as the main pollutants discharged from the industrial area.

The water quality sampling locations is shown in Figure 2-5.

Figure 2-5: Water Quality Sampling Locations

From the primary test result, BOD5 was found as 1 mg/l (dry season) at both sampling locations (SW-01-IWM and SW-02-IWM) of Ichakhali khal. After the analysis of raw water data of samples collected from Ichakhali khal in three batches (Table 2-3 and Table 2-4), it was found that the wet and dry season BOD5 are 3.6 and 1.6 mg/l respectively (BEZA ,2019a).

Table 2-3: Characteristics of raw water collected in the first three batches (wet season) from Ichakhali Khal at Jhulanpul Bazar location

| Sl. No. | Water Quality Parameter | Unit | Concentration Present |  |  | Bangladesh Drinking Water Standard |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Batch 1 | Batch 2 | Batch 3 |  |
| 1 | pH | - | 7.14 | 7.07 | 7.34 | 6.5-8.5 |
| 2 | Color (Apparent) | Pt-Co | 1084 | 361 | 194 | 15 |
| 3 | Color (True) | Pt-Co | 130 | 201 | 33 | 15 |
| 4 | Turbidity | NTU | 190 | 152 | 78.3 | 10 |
| 5 | Total Dissolved Solids (TDS) | mg/L | 75 | 71 | 133 | 1000 |
| 6 | Iron (Fe) | mg/L | 3.2 | 4.8 | 2 | 0.3-1.0 |
| 7 | Total Coliform (TC) | CFU/100 mL | 26000 | TNTC | TNTC | 0 |
| 8 | Fecal Coliform (FC) | CFU/100 mL | 17400 | TNTC | TNTC | 0 |
| 9 | Electrical Conductivity (EC) at 25oC | µS/cm | 113 | 136 | 247 | -- |
| 10 | Dissolved Oxygen (DO) | mg/L | 5.95 | 5.31 | 5.48 | 6 |
| 11 | Ammonia (NH3-N) | mg/L | 0.55 | 0.8 | 1.48 | -- |
| 12 | Total Suspended Solids (TSS) | mg/L | 153 | 119 | 97 | 10 |
| 13 | Temperature | oC | 29.6 | 29.5 | 27.7 | 20-30 |
| 14 | Chemical Oxygen Demand (COD) | mg/L | 10 | 11 | 13 | 4 |
| 15 | Biochemical Oxygen Demand (BOD5) | mg/L | 2.8 | 2.8 | 3.6 | 0.2 |
| 16 | Chlorophyll-a | µg/L | 3.7 | 5.1 | 0 | -- |

Source: BEZA (2019a)

Table 2-4: Characteristics of raw water collected in the dry season (Batch 4, 5, and 6) from Ichakhali Khal at Jhulonpul Bazar location.

| Sl. No. | Water Quality Parameter | Unit | Concentration Present |  |  | Bangladesh Drinking Water Standard |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Batch 4 | Batch 5 | Batch 6 |  |
| 1 | pH | - | -- | 7.26 | 7.49 | 6.5-8.5 |
| 2 | Color (Apparent) | Pt-Co | -- | 150 | 197 | 15 |
| 3 | Color (True) | Pt-Co | -- | 64 | 122 | 15 |
| 4 | Turbidity | NTU | -- | 26.3 | 39.2 | 10 |
| 5 | Total Dissolved Solids (TDS) | mg/L | -- | 129 | 136 | 1000 |
| 6 | Iron (Fe) | mg/L | -- | 0.70 | 1.28 | 0.3-1.0 |
| 7 | Total Coliform (TC) | CFU/100 mL | -- | TNTC | TNTC | 0 |
| 8 | Fecal Coliform (FC) | CFU/100 mL | -- | TNTC | TNTC | 0 |
| 9 | Electrical Conductivity (EC) at 25oC | µS/cm | -- | 221 | 216 | -- |
| 10 | Dissolved Oxygen (DO) | mg/L | -- | 4.44 | 4.18 | 6 |
| 11 | Ammonia (NH3-N) | mg/L | -- | 0.38 | 0.35 | -- |
| 12 | Total Suspended Solids (TSS) | mg/L | -- | 27 | 32 | 10 |
| 13 | Temperature | oC | -- | 22.1 | 28.8 | 20-30 |
| 14 | Chemical Oxygen Demand (COD) | mg/L | -- | 7 | 5 | 4 |
| 15 | Biochemical Oxygen Demand (BOD5) | mg/L | -- | 1 | 1.6 | 0.2 |
| 16 | Chlorophyll-a | µg/L | -- | 0.9 | 6.8 | -- |

Source: BEZA (2019a)

### Forecast Result

The BOD5 concentration (2040) after complete mixing of Ichakhali khal water and wastewater of Zone 2A and 2B is given in Table 2-5 and Table 2-6. It is evident that the BOD concentration in flow of Ichakhali Khal will increase to 23-24 mg/l even after the treatment of wastewater to allowable limit of 40 mg/l. The concentration is way above the limit of BOD in river water for fish of 6 mg/l and for irrigation of 10 mg/l.

Table 2-5: BOD5 Concentration after Complete Mixing (2040) in Ichakhali Khal for 80% Dependable flow (m3/s)

| Period | Flow in Ichakhali Khal (m3/s) | Present BOD Conc. in Ichakhali Khal (mg/l) | Wastewater Flow (m3/s) | Wastewater BOD Conc. (mg/l) | Flow after complete Mixing (2040) (m3/s) | BOD Conc. after complete Mixing (2040) (mg/l) |
| --- | --- | --- | --- | --- | --- | --- |
| Dry (Dec-Feb) | 0.7 | 1 | 0.935 | 40 | 1.64 | 23.30 |
| Wet (Jun-Oct) | 0.7 | 3.6 | 0.935 | 40 | 1.64 | 24.42 |
| Average | 0.7 | 2.3 | 0.935 | 40 | 1.64 | 23.86 |

Table 2-6: BOD5 Concentration after complete Mixing (2040) in Ichakhali Khal for 50% Dependable flow (m3/s)

| Period | Flow in Ichakhali Khal (m3/s) | Present BOD Conc. in Ichakhali Khal (mg/l) | Wastewater Flow (m3/s) | Wastewater BOD Conc. (mg/l) | Flow after complete Mixing (2040) (m3/s) | BOD Conc. after complete Mixing (2040) (mg/l) |
| --- | --- | --- | --- | --- | --- | --- |
| Dry (Dec-Feb) | 11.9 | 1 | 0.935 | 40 | 12.84 | 3.84 |
| Wet (Jun-Oct) | 2.2 | 3.6 | 0.935 | 40 | 3.14 | 14.46 |
| Average | 7.05 | 2.3 | 0.935 | 40 | 7.99 | 9.15 |

References for Appendix D

- Yanmar Japan Thilawa Development Ltd. (2016), Thilawa Special Economic Zone Development Project (Zone B)

- BEZA (2019a), Draft Final Report on Detail Study on Total Water Demand & Water Availability Assessment for BSMSN, Institute of Water Modelling (IWM)

- WHO (1993), Assessment of Sources of Air, Water and Land Pollution

- BEZA (2019b), Development Program and Transport Assessment for BSMSN, Sheltech

- Stein, A.F., Draxler, R.R, Rolph, G.D., Stunder, B.J.B., Cohen, M.D., and Ngan, F., (2015). NOAA's HYSPLIT atmospheric transport and dispersion modeling system, Bull. Amer. Meteor. Soc., 96, 2059-2077, http://dx.doi.org/10.1175/BAMS-D-14-00110.1

- Rolph, G., Stein, A., and Stunder, B., (2017). Real-time Environmental Applications and Display System: READY. Environmental Modelling & Software, 95, 210-228, https://doi.org/10.1016/j.envsoft.2017.06.025

Appendix E: Associated Facilities Screening Form

Table: Identification of Potential / Likely Associated Facilities for ______________ Project

| Infrastructure | Description | Directly related to Project | Contemporaneous | Necessary for the project to be viable | Would NOT be constructed without PRIDE project | Associated Facility as per ESF defn.? | Implementing Agency | Status | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Notes:

- For an activity to become an associated facility, all of the above criteria need to be fulfilled.

- Few other facilities may be found meeting the criteria of Associated Facilities which will be put in the table as soon as relevant.

Appendix F: Traffic Management Plan

### 1.1	Objective of the Plan

The objective of this plan is to provide guidance for management of traffic during construction and operation phase. This plan should be followed for management and smooth flow of the traffic and to prevent the accidents during construction and operation phase of the project.

### 1.2	Current scenario of traffic in project area

At present project site is accessible through non-motorable Embankment constructed as boundary/dike for Mirsarai EZ-I. A single lane road is being constructed on this Embankment for Mirsarai EZ site I. For Mirsarai EZ-II, it is proposed to widen the road to double lane to accommodate the traffic from EZ-II. This road will connect site to N1 (Dhaka Chittagong Highway) through Abu Torab Road. The road is proposed to be widened in future by GoB.

Traffic is increasing every year on this N-1. Bi-directional traffic volume on the highway in 2009 was recorded to be 56, 32,798 whereas it was 32, 06,277 in 2006. Average traffic growth rate of 21.03% is estimated as per the study carried out by BUET. Thus at present traffic with this growth rate at this highway is expected to be 5,75,12,557. The highway is now being expanded from 2 lanes to 4 lanes which has improved the traffic condition drastically and has reduced travel time between the two cities (Dhaka & Chittagong).

### 1.3	Roles and Responsibility

EHS cell head, environmental & safety officers along with traffic Marshals will be responsible for safe traffic operation during the construction and operation phase of the project. Roles and responsibility is defined below.

Table 1: Roles and responsibility for traffic management

| Activity | Responsibility and Supervision |
| --- | --- |
| Construction Phase |  |
| Managing hours of vehicle movement carrying material and debris (non-peak hours), designating the route & marking the ways | Environment and safety officers & EHS cell head |
| Provision of boards and signage for guiding the way and instruction for speed limits, no honking & safe driving practice along the access road and haulage roads | Environment and safety officers & EHS cell head |
| Provision of self-defensive training to rivers | Environment and safety officers & EHS cell head |
| Provision of traffic management training to traffic marshals | Environment and safety officers & EHS cell head |
| Checking the records of the vehicles enter and left from the register kept at entry gate | Environment and safety officers & EHS cell head |
| Ensuring vehicles and pedestrians are following the designated paths | Traffic Marshals & Environment and safety officers |
| Minimizing the vehicular movements within the site | Traffic Marshals & Environment and safety officers |
| Ensuring parking of the vehicle and construction equipment at designated place | Traffic Marshals & Environment and safety officers |
| Turning vehicle, sign and no honking at site | Traffic Marshals & Environment and safety officers |
| Ensuring all vehicles entering the premises carry PUC | Traffic Marshals & Environment and safety officers |
| Operation Phase |  |
| Survey of vehicles on the access road to the project to know the traffic count and the condition of the road | Environment and safety officers |
| Ensuring the signage are provided everywhere for safe driving practices, speed limits, directions | Environment and safety officers |
| Ensuring the parking lots are being used for parking and no parking is being undertaken at unauthorized location | Traffic Marshals & Environment and safety officers |
| Checking entry and exit register of the vehicles | Traffic Marshals & Environment and safety officers |
| Ensuring all vehicles entering the premises carry PUC | Traffic Marshals & Environment and safety officers |

### 1.4	Traffic Control Devices

Traffic control device used to regulate the traffic in Road Construction Zones include

- Road sign

- Delineators

- Barricades

- Cones

- Pylons

- Pavement marking

- Audio visual flash lights

- Electric rope lights

Suggested plan for designing signage at minimum sightline distance is given below

| Average speed (km/hr) | Distance of first sign in advance of the first channelizing device (m) | Size of Warning Sign (mm) | Minimum no of signs in advance of the hazard |
| --- | --- | --- | --- |
| Under 50 | 100 | 600 | 3 |
| 51-60 | 100-300 | 750 | 3 |
| 61-80 | 120-300 | 900 | 3/4 |
| 81-100 | 300-500 | 1200 | 4 |
| Over 100 | 1000 | 1200-1500 | 4 |

### 1.5	Dress code and necessary qualification of Flagmen

Flagmen are required for guiding and maintaining the flow of the traffic. Flagmen should have some minimum qualification and should be mentally alert and good in physical condition. Dress code of the flagmen should be as follows

- Yellow helmet with green reflective sticker fixed around and reflective jacket along with hand signaling devices such as flags/batten Lights/sign paddles.

- Flagman shall stop the traffic for a short while whenever required (e.g. For entry and exit of construction equipment in to work zone)

- Flagman should be positioned in a place where he is clearly visible to approaching traffic and at a sufficient distance to enable the drivers to respond for his flagging instructions.

- A flagman never leaves his post until properly relieved

- The standard distance shall be maintained at 60 – 100 m but can be altered depending upon the approach speed and site conditions. In urban areas this distance shall be taken as 20 m to 50 m.

## 1.6	Traffic Management for Construction Phase

Construction equipment and vehicle expected at the site majorly includes Road rollers, graders, compactors, excavators (back hoe), hydra/loading truck, transit mixers, crane, dumpers and trucks. Following things will be checked for every equipment and vehicle and driver

- Valid Driving license

- PUC Certificate

- Maintaining speed limits (10 kmph within site and 15-20 kmph outside site)

- Vehicle fitness (brakes, mirrors, lights, wind screen, indicators, horns, exhaust, tyres etc.)

- Parking of vehicle to be undertaken at authorized location and for authorized vehicle

- Following of designated path by the vehicle within the site

- Fastening of Seat belt for the driver and the person sitting in the front seat of a vehicle is a must. Drivers of cranes, fork lift trucks and similar vehicles are exempted from this requirement of the seat belt. If seat belt is available in the back seat, this must be used

- No overtaking of any vehicle is permitted inside & outside of construction site. However heavy vehicles like crane, tractors, dumpers and others can be overtaken with proper warnings, signals and considering the movement of the traffic nearby.

- Use horn to warn the nearby traffic judiciously. It is desirable to blow horn in 2 or 3 small blasts instead of one full blast so that traffic ahead may not panic and lead to an accident. Condition of the horn should be perfect and can be checked as an item of daily checklist. Every vehicle must have a reversing horn.

- No speed breakers within the site but rumble strips will be provided to check the speed

- No driver noted consuming alcohol, drugs, sleeping pills, anti histamins and other item will be allowed within the site

- Drivers should be given self-defense training from time to time which should focus on do’s and don’ts for the drivers like regarding alertness, awareness, driving during bad weather conditions & night time and safety guidelines.

Apart from the above checklist items, a system of permit should be developed at construction site. The vehicle should be given permit to enter within the site only if it meets the following requirement

- All the vehicles entering the site should be in good condition which is ensured by security department

- Any vehicle entering the site shall undergo necessary security check as per security requirements and will be issued a vehicle access permit

- All permanent vehicles, which are required toplyin the site regularly, are issued a VAP for specific period decided by the security department.

- All temporary vehicles also will be issued VAP as per security guidelines

- The vehicle access permit should be visibly displayed on the windshield

- All vehicles involved in the excavation and/or demolition process and departing the property with demolition materials, spoil or loose matter must have their loads fully covered before entering the public roadway

- The vehicle entering in the site can be inspected by the environment and safety officer

### 1.7	Special guidelines for heavy construction equipment and vehicles

All the heavy vehicles like cranes, trucks, tractors and others should be run in a safe manner so other traffic is not exposed to any accidental hazard.

- Proper warning signals like a flag of red cloth and lights on all the sides of the vehicle to guide nearby traffic.

- All cranes above75 tones capacity should be protected in the front as well as back to guide the traffic on the road.

- Speed should be maximum 10 km/ph and at turning proper care should be taken

- Vehicle entry permits to be raised for entry into sites.

- The driver of heavy vehicle must report to the security guard at the entry gate. The guard shall check the necessary details and if satisfied, then allows the entry. These checks however shall not relieve the driver from his responsibilities of meeting other legal provisions that may cover the transportation of such vehicles.

- Cargo on trucks and trailers should be properly secured so that they should not fall out from truck / trailer. No material should protrude out of truck, which can cause injury or damage to property

- Whenever a crane is moving from one place to another, the boom or jib must be hooked onto the crane's body. When not in use, all cranes must be parked in a safe manner

### 1.8	Pedestrian Safety

Pedestrians are exposed to many dangers on the road and have become victims of many accidents. On a road, the maximum danger is to the pedestrian. The following are some basic rules to be complied with by pedestrians and traffic managers at site for avoiding any accident.

- Traffic marshals and flagmen at site should ensure that pedestrian always walk on the edge of the road, marked by white line or on the designated routes only and keep vehicles off from pedestrian walkway

- If no marking, walk single file on the extreme edge of the road facing the oncoming traffic, so that you can keep a watchful eye on the traffic and not get hit from behind.

- Keep your eyes on pot holes and other obstacles in your way.

- While crossing the road use zebra crossing wherever available. If a zebra crossing is not available, choose a safe point where traffic is clearly visible on both sides. Never cross at a blind curve.

- At an intersection, never try to cross diagonally.

### 1.9	Traffic Management for Operation Phase

It is anticipated traffic of app. 600 PCU will increase after project development. Thus it is required to manage the traffic to prevent congestion and the accidents on access roads and in internal roads. Following should be done for managing the traffic

- Position of flagmen at required locations like entry/exit, turns, diversions etc.

- It should be ensured that signage is provided adequately regarding directions, speed limits, diversions, turns etc.

- Speed on access roads and internal roads should be 10-15 kmph

- Vehicle entering the site should be checked for fitness and availability of PUC prior entering the site by traffic marshal/environment or safety officers

- Use of horn should be prohibited at the site

- Parking area should be kept free always and no vehicle should be allowed to park on road

- Every industry should provide dedicated loading and unloading area to prevent parking of vehicle on road

- Checklist for vehicle entering the site should be maintained

- Accident in case, if it occurs should be recorded and reported in monthly, SHE reports

APPENDIX G: WASTE MANAGEMENT PLAN

## Introduction

All project generated wastes will need to be managed and disposed of in a manner to prevent potential impacts on the environment and risks to human health. A Waste Management Plan (WMP) for the proposed project has been developed by following the World Bank’s Environmental Social Standard (ESS-3).

## Objectives

The construction and operation of the proposed project will generate various type of waste which will need appropriate collection, transportation, primary treatment, and disposal. Hence, to serve the purpose, a Waste Management Plan has been formulated to demonstrate:

- Inventory of waste in different type of categories like garbage, rubbish, hazardous, waste etc.;

- Keep the site in a clean and tidy state to reduce the attraction of pest species, impacts on the local environment and negative impacts on visual amenity; and

- The suggestion of options for waste handling and disposal during construction and operation phase of the project

## Scope

This plan shall be applicable to the contractor engaged by BEZA for the construction phase of the proposed project. The elements of the plan will be directly implemented by the contractors hired by the BEZA. All of the work will be monitored by the EHS officer of the BEZA.

## Roles and Responsibilities

Site Supervisor of Contractor

Site Supervisor will be responsible for the following activities:

- Management of onsite waste generation associated with construction works to help avoid excessive generation where practicable;

- Keeping of all records of waste type which are construction waste and debris, hazardous waste;

- Liaison with government approved vendors for disposal of hazardous waste generated;

- To have authorization for hazardous waste generation and storage granted by the government in place.

EHS Manager of Contractor

The following responsibilities are entrusted to the EHS Manager:

- Demarcation of area within the project area for keeping of segregated wastes;

- Labelling of the drums containing hazardous wastes like used oil;

- Maintaining of receipts for hazardous waste management records;

- Notifying the Site Supervisor of any activity that may generate a large amount of waste to allow appropriate controls to be put in place to manage waste generated;

EHS Manager of BEZA

EHS officer of BEZA will cross check the assigned responsibilities of Supervision and EHS of contractor.

## Waste Types and Quantities Generated

All waste generated from the project will be categorized as either non-hazardous or hazardous following an assessment of the hazard potentials of the material, in line with local and national requirements.

### Construction Phase

The waste will generate from construction activities like site clearing, leveling etc. Other categories of waste will be produced daily and comprise of the following:

- Dredged material;

- Soil waste;

- Food waste from kitchen premises of labour camps;

- Construction debris; and

- Sewage from temporary toilets;

The construction and decommissioning phases will require the use of hazardous materials such as diesel or petrol to supply the fuel equipment and vehicles and maintain equipment. The following hazardous wastes will also be produced from construction activities.

- Oily rags;

- Used oil and oil filters - from generators or vehicle maintenance; and

- Scrap and packaging material

### Operation Phase

Operations and maintenance of the on-site infrastructures of EZ are not expected to generate any significant amount of waste until start the industrial operation at EZ. Thus, operation of onsite facilities will only generate organic waste like Food waste from kitchen premises, Sewage from toilets etc. However, operation of EZ with industry can produce following hazardous waste

- Fuel requirements like greasing, transformer oil etc.

- Used oil; and

- Industrial untreated effluent

## Selection of Disposal Sites

To manage the generated waste from both phases a designated place need to be selected. The locations of Disposal sites have to be selected such that:

- Disposal sites are located at least 1000 m away from sensitive locations like settlements, water body, notified forest areas, wildlife/bird/dolphin sanctuaries or any other sensitive locations.

- Disposal sites shall not contaminate any water sources, rivers, sea etc. so the site should be located away from water body and disposal site should be lined properly to prevent infiltration of water.

- Public perception about the location of debris disposal site has to be obtained before finalizing the location.

- Permission from the village/local community is to be obtained for the Disposal site selected.

- Environment Engineer of PMC and Executive Engineer of Contract Management Unit must approve the Plan before commencement of work.

## Waste Handling, Management, and Disposal

### Construction Phase

All wastes produced from the project activities on site will be temporarily stored in designated waste storage areas. All wastes that cannot be reused or recycled will be collected by approved waste contractors and transferred to an appropriately licensed waste management facility for treatment and disposal. Following steps will be taken to manage the waste generated during construction phase:

- Fuel will be stored on-site in temporary aboveground storage tanks and will be stored in a locked container within a fenced and secure temporary staging area;

- Trucks and construction vehicles will be serviced off-site;

- The Contractor will at all times ensure that the entire existing canal and drains within and adjacent to the site are kept safe and free from any debris.

- While disposing debris / waste material, the Contractor will take into account the wind direction and location of settlements to ensure against any dust problems.

- All concrete mixing be undertaken on impermeable plastic lining to prevent contamination of the soils and surrounding areas;

- Food waste and other refuse are to be adequately deposited in sealable containers and removed from the kitchen frequently to avoid accumulation;

- The use, storage, transport and disposal of hazardous materials used for the project will be carried out in accordance with all applicable regulations;

- All hazardous waste to be disposed of off to government approved vendors;

- Material Safety Data Sheets for all applicable materials present on site will be readily available to onsite personnel;

- All construction debris will be placed in appropriate on-site storage containers and periodically disposed of by a licensed waste contractor;

- The construction contractor will remove refuse collected from the designated waste storage areas at the site at least once a week and

- Empty fuel containers will also be stored at a secured area designated for scrap and sold to authorized vendors. All packaging material will also be collected at the storage area and sold to scrap dealers

- Contractor should display the board at disposal site stating the name of project, usage of the site and type of debris being disposed.

- A guard shall be kept at disposal site to prevent any unauthorized disposal of waste at the debris disposal site

### Disposal of Dredged Material

Dredging shall be carried out in the sea for obtaining sand for filling the land. Dredged sand should be as per the requirement only. Excess dredged material shall not be disposed off near the shore or along the Ichakhali canal. Dredged material should be tested for toxicity prior usage for filling purpose. No dredging should be carried out during spawning period of Hilsa (September to October) and should strictly be avoided during 15-24 October which is peak season for Hilsa spawning (Fifth National Report of Bangladesh to the Convention on Biological Diversity, DoE, MoEF, GoB).

## Record Keeping

Site approved by site engineer only can be used as disposal site. Record of all such site should be maintained along with the area of disposal site, type & quantity of material disposed daily and capacity of disposal site.

## Guidelines for Rehabilitation of Disposal Sites

The dumpsites filled only up to the ground level could be rehabilitated as per guidelines below and to be decided by the Engineer and the supervision consultant.

- The dumpsites have to be suitably rehabilitated by planting local species of shrubs and other plants. Local species of trees has also to be planted so that the landscape is coherent and is in harmony with its various components.

- In cases where a dumpsite is near to the local village community settlements, it could be converted into a play field by spreading the dump material evenly on the ground. Such playground could be made coherent with the landscape by planting trees all along the periphery of the playground.

- Closure of the disposal site should be up to the satisfactory level of site engineer

## Penalties

Stringent action & penalties should be imposed off on contractor for dumping of materials in locations other than the pre-identified locations. Grievance Redressed mechanism should be in place for taking note and action on such complaints.

# Appendices H: Ecological Baseline

## Livestock & Poultry

Figure 1.1-1: Photographs Showing Livestock & Poultry in the area

## Fish and Shrimp Species Recorded in the Feni River Estuary

Table 1: Fish and Shrimp Species Recorded in the Feni River Estuary

| Family | Scientific Name | Local Name | Habitat | Climate |
| --- | --- | --- | --- | --- |
| Ambassidae | Chanda nama | Chanda | Benthopelagic Freshwater brakish | Tropical |
|  | Stolephorus sp. | Mola | Benthopelagic Freshwater brakish | Tropical |
| Anguillidae | Anguila sp. | Kuicha | Marine, brakish | Tropical |
| Alpheidae | Alpheaus spp. (Rafinesque, l8l5) | Alphaed shrimp | Marine, brakish | Tropical |
| Bagridae | Mystus gulio(Hamilton, l822) | Guilla | Demarsal, anadromous, freshwater, brakish | Tropical |
| Clupeidae | Tenualosa toil (Valenciennes, l847) | Illish | Marine, freshwater, Brakish, Pelagic- neritic, anadromous | Subtropical |
| Cynoglossidae | Cynoglossus lingua | Kukur jeeb | Demarsal, amphidromus, Freshwater, brakish, marine | Tropical |
| Cyprinidae | Puntius ticto | Tit punti | Benthopelagic, Freshwater, | Tropical |
| Engrualidae | Coila ramkorati (Hamilton, 1822) | Alua | Pelagic- neritic, amphidromus, brakish, Marine | Tropical |
| Gobiidae | Apocryptus bato (Hamilton, 1822) | Chiring | Demarsal, Amphidromus, freshwater, brakish, Marine | Tropical |
|  | Pseudapocryptes lanceolatus (Bloch- Schneider,1801) | Goby | Amphidromus, freshwater, brakish, Marine | Subtropical |
|  | Odontamblyopus rubicundus (Hamilton, 1822) | Raja Cheoa | Marine brackish, benthopelagic, amphidromous | Subtropical |
|  | Oxyurichthys microlepis | Nunabaila | Marine brackish, | Tropical |
| Leucosiidae | Matuta victor (fabricus,1781) | Kakra | Marine brackish, | Tropical |
| Mugilidae | Mugil cephalus (Linnaeus,1758) | Bata | Benthopelagic Amphidromus, freshwater, brakish, marine | Subtropical |
| Mastacembelida e | Mastacembelus armatus | Baim | Marine, Brackish, Freshwater | Subtropical |
| Polynemidae | Polynemus peradiseus (Linnaeus, 1758) | Taposi | Marine, freshwater, brackish, demersal, amphidromous | Tropical |
| Sciaenidae | Johnius belangerii (Cuvier, 1830) | Poa | Demarsal amphidromus brakish, marine | Tropical |
| Taenioididae | Trypauchen vagina | Lal cheoa | Marine brackish, benthopelagic, amphidromous | Subtropical |
|  | Pangassius pangassius | Pangass | Freshwater, brakish, | Subtropical |
| Palaemonidae | Macrobrachium rosenbergii | Goldachingry | Fresh water, Esturine water | Subtropical |
|  | Macrobrachium villisimanus | Dimuaicha | Brackish, | Tropical |
| Peneaidae | Penaeus monodon | Bagda chingri | Marine brakish | Subtropical |
|  | Parapenaeopsis sculptelies | Boro chama | Marine, brakish | Tropical |
|  | Matapenaeus monoceros | Horina chingri | Marine, brakish | Tropical |
|  | Parapenaeopsis stylifera |  | Marine, brakish | Subtropical |
| Palaemonidae | Exopalaemon stylifera |  | Marine, brakish | Subtropical |
|  | Macrobrachium mirabile |  | Marine, brakish | Subtropical |
| Portunidie | Scylla serrata | Kakra |  | Subtropical |

## Trees species observed beside access roads of the project area

Table -1: List of road site planted tree species

| Species Name | Local Name | Family |
| --- | --- | --- |
| Acacia moniliformes | Akashmoni | Leguminosae |
| Areca catechu | Supari | Palmae |
| Avecenia alba/ Avicennia Marinavierh | Baen | Aviceniaceae |
| Azadirachta indica | Neem | Meliaceae |
| Carica papaya | Pepey | Caricaceae |
| Cocos nucifera | Narikel | Palmae |
| Excoecaria agallocha | Gewa | Euphorbiaceae |
| Herritiera fomes | Sundari | Sterculiaceae |
| Mangifera indica | Aam | Anacardiaceae |
| Manilkara zapota | Safeda | Zapotaceae |
| Phoenix sylvestris | Khejur | Palmae |
| Psidium guajava | Peyara | Myrtaceae |
| Sonneratia apetala | Kewda/Keora | Lyrthaceae |
| Spondius pinnata | Amra | Anacardiaceae |
| Swietenia mehagoni | Mehogani | Meliaceae |
| Syzygium cumini | Jaam | Myrtaceae |
| Syzygium samarengense | Jamrul | Myrtaceae |
| Tamarindus indica | Tentul | Leguminosae |
| Terminalia catapa | Kathbadam | Combrataceae |
| Tectonia grandis | Teak | Lamiaceae |
| Gmelina arborea | Gamari | Verbenaceae |
| Brassica juncea | Raisharisha | Brassicaceae |
| Olea europaea | Olives | Oleaceae |
| Terminalia bellirica | Bohera | Combretaceae |
| Terminalia chebula | Horitoki | Combretaceae |
| Phyllanthus emblica | Amla | Phyllanthaceae |
| Protium serratum | Gutgutiya | Burseraceae |
| Propospis cineraria | Jand | Fabaceae |
| Casuarina equisetifolia | Jhao Ghas | Casuarinaceae |
| Albizia lebbeck | Koroi | Fabaceae |
| Delonix regia | Krishnachura | Fabaceae |
| Leucaena leucocephala | Ipil Ipil | Fabaceae |
| Carissa carandas | Karamcha tree | Apocynaceae |
| Artocarpus heterophyllus | Jack fruit | Moraceae |
| Polyalthia longifolia | Pseudo Ashoka | Annonaceae |
| Musa parasisiaca | Banana | Musaceae |
| Garuga pinnat | Bhadi | Burseraceae |
| Abelmoschus esculentu | Bhendi | Malvaceae |
| Erythrina indica | Mandar | Fabaceae |
| Acacia mangium | Mangiam | Fabaceae |
| Dalbergia sissoo | Sheeshu | Fabaceae |
| Lawsonia inermis | Kat Mehndi | Lythraceae |
| Swietenia mahagony | Mahagony | Meliaceae |
| Arundinaria | Cane Ghas | Poaceae |
| Samanea saman | Rain Tree | Fabaceae |
| Zizyphus mauritiana | Boroi | Rhamnaceae |
| Calotropis | Calotropis | Apocynaceae |
| Ceriops decandra | Gora | Rhizophoraceae |
| Litchi chinensis | Litchi | Sapindaceae |
| Ficus benjamina | Dumur | Moraceae |
| Citrus maxima/Citrus grandis | Jambura | Rutaceae |
| Mimosa pudica | Lazzabati | Fabaceae |
| Pinus densiflora | Pine | Pinaceae |
| Excoecaria agallocha | Gewa | Euphorbiaceae |
| Nypa fruticans | Golpata | Arecaceae |
| Heritiera littoralis | Sundri | Lauraceae |

Source: Field Visit

Figure -1: Photographs Showing road site planted tree species

## Buffer Zone: Ecological Assessment

### Agro-Ecological Zones within the Project Area

The physiographic unit of the project is: Chattogram Coastal Plain (AEZ# 23), sub-regions are Piedmont Plains and River Flood plain (23a) and Young Tidal Floodplain(23b) with general soil type Non- Calcareous Grey Floodplain Soils(non-saline) in both of the sub-region soils. The soil texture in both of the sub-region soils is loamy. The landscape of the area-comprised of mainly medium high land to high land. The soil type pre-dominates with Non-Calcareous Grey floodplain soils(no-saline). The Agro-ecological zones of Bangladesh are demonstrated in Figure -1.

### Farming Practice

Agriculture

Farmers practice agriculture in this region for 3 seasons, i.e. Rabi, Kharif I and Kharif II. Source of irrigation in this region are storm water are rain water harvesting ponds, ground water, khals/canal etc. Major crops of the region are paddy, betel leaf, betel nut, potato, corn, turmeric, tea, peanut, mustard, patol (heap), brinjal, ginger, cucumber and other vegetables.

Aquaculture

During the 1960s, coastal aquaculture in the Chattogram region was normally a function of salt production. During the monsoon season, some salt producers would utilize their salt pans mainly for subsistence fishing but also occasionally to supplement their income through the sale of the shrimp and finfish surplus. With the increase of demand and price, this practice gained increased acceptance in the region. Harvesting of tiger shrimp is most practiced due to its demand.

Aquaculture is practiced extensively around the EZ site. Govt. has initiated various aquaculture programs to harvest fishes especially Ruhi, Catla and Korps near the EZ site for providing the livelihood to the people in Mirsarai. Fising is being done intensively in River Feni and Sea. Fishing is done in Ichakhali Channel and Bamkn Sundar Channel during high tides and monsoon season.

Shrimps culture (Bagda and Golda) is also practiced in large scale. The culture period was typically four months for Bagda and six to seven months for Golda. Varieties of feeds such as cooked rice, fishmeal, oilcake and snail muscle are used for shrimp culture. Shrimp culture in the area is affected by some viral disease which attacks shrimp culture and are responsible for the great loss of shrimp in this area. Mud crabs are also collected by people from the mudflats area, i.e. forest and along the canals.

Source: BEZA (Nov 2016 a)

Figure -1: Agro-Ecological Zones of Bangladesh

### Cropping Pattern and Intensity

Major portion of the land within study area is under agriculture. Agriculture land comprise of fallow and agriculture land areas, seasonal gher and aquaculture ponds. There are three cropping seasons, i.e. Kharif-I, Kharif-II and Rabi. Major agriculture pattern in study area is Fallow-T.Aman (Local), local vegetables & fruits and B. Aus. Major crops of each cropping season are given in Table 2.

Table -1: Cropping Seasons in Area

| Sl. No. | Cropping Season | Major Crops | Months |
| --- | --- | --- | --- |
| 1. | Kharif-I | B.Aus, Jute and vegetables | March to May |
| 2. | Kharif-II | T.Aman (HYV & L) and fallow land | June to October |
| 3. | Rabi Season | Boro (HYV), vegetables, fruits and pulses | November to February |

Source: FAO, Bangladesh

Figure 1.4-2: Photographs Showing Agriculture Fields and Aquaculture Ponds

Crop Calendar

During Kharif 1 season HYV rice is the principal crop, during Kharif 2 medium Aman and Aus crops are grown or jute is cultivated in medium lowland. During the Rabi season, wherever the land dried up in early October to mid-October, farmers grow legumes, oil seed and vegetables. Vegetables are mostly confined to highland, oil seeds to medium land and pulses to medium lowland. Paddy is major cash crop of the farmers in the study area and requires year-round production activity. Oilseeds, vegetables and other seasonal crops are the significant crops in this area. Cropping patterns and crop diversity for kharif and Rabi seasons are scheduled according to land types, flooding frequency and duration of floods. The patterns in the study area are dictated by the local hydrologic regime and are essentially based on rice. Non-rice crops are grown mostly in the non-irrigated land in rotation with Rabi Aus and Aman, depending on the land type. Winter crops are intercropped with sugarcane. Irrigated HYV Boro is grown is winter in sequence with T. Aman on medium high land and T. Aman on medium lowland. Major vegetables in the area are tomoato, potato, lady finger, brinjal, pumpkin, cabbage, cauliflower, radish, sweet potato, jackfruit etc. Legumes grown are arhar, masor, keshari and moong dal.

### Cropped Area

As per the land use land cover study of 10 km radius of the project site, approx. 29.3% of the area is covered under agriculture land& aquaculture ponds. No agriculture land area will be used for development of EZ.

### Crop Production, Damage and Constraints of Crop Production

Farmers of Chattogram district produce food crops, cash crops, fruits, vegetables, livestock and poultry, fish, timber and fuel wood. Majority of household also have poultry, livestock and produce poultry and livestock produce for daily consumption and selling.

Major constraints of agriculture in the study area is availability of arable land, crop damage, seasonal flooding of land, water logging, soil salinity and availability of high quality seeds and fertilizers. Crop damage is reported to occur during focused group discussions due to both excess and scanty rainfall. Pre- monsoon and post-monsoon drainage congestion limits crop production. In monsoon season, the duration of water logging thus limiting the crop choice.

### Common Flora and Fauna in buffer zone

10 km radius area around EZ site majorly comprises of agricultural/aquaculture land followed by water bodies. Flora and fauna in the study area is given below. Ramgarh reserve forest & Mahamaya forest are other zones rich in bio-diversity but are not within 10 kms radius of EZ site.

### Terrestrial Flora in Buffer Zone

There are no major forests apart from Domkhali forest, Bamon Sundar forest & Moghadia forest occurs within 10 km radius area of EZ site. Flora in the study area majorly comprises of the trees which were found along the access road and are given in the table above. Apart from the trees mentioned in the above table trees existing in study area are: Deodara (Cedrus deodara), yellow bell (Tecoma stans), Hibiscus (Hibiscus rosa sinensis), Chikrashi (Chukrasia tabularis), Telsur (Butea monosperma), Jarul (Lagerstroemia speciosa), Kadam (Neolamarckia cadamba), Dhakijam (Syzygium grande), Davana (Artemisia pallens), Lotkon (Baccaurea ramiflora), money plant (Epipremnum aureum), Toon (Toona ciliate), Bokain (Melia azedarach) and Banyan (Ficus benghalensis). Photographs of flora of buffer zone aregiven below in Figure-3. Varied variety of vegetation occurs along the Ichakhali and Bamon Sundar canal in study area. This vegetation is both naturally occurring and planted by forest department. Golpata is planted all along the Bamon Sundar canal by Forest Department, Mirsarai.

Figure-3: Photographs Showing Vegetation in Buffer Zone

### Terrestrial Fauna in Buffer Zone

Mammals & Reptiles

Cows, goats, dogs, cats, mule, horse, monkey, fox, deer are found in the study area during the visit. No significant wild fauna was found in study area. As per discussion with local people it was learned that deer and fox are found in planted mangrove forests but none were spotted during visit.In Ramgarh reserve forest wild animals like Fox, Monkeys, Langoor, Ullunk, Wild Cats, Wild Boar, Bisons, Deer, Otter/Udbilaw (Lontra Canadensis), Kat Biral, Elephant, Bonrui, Rabbits, Deers, Wild goats & wild goats. But Ramgarh forest does not falls within 10 km radius area. Apart from the mammals, reptiles like chameleon, garden lizard& Gohar Saanp were also observed during the visit. Apart from this villager informed that cobras and python also present in this region.

Avifauna

Avifaunalike Gugu/Dove (streptopelia chinensis), Paira/Pigeon, Doyal (Magpie Ribbon), House sparrow/Choroi, Parrot/Tiya, Crow (Corvus splendens), Myna/Shalik, Babui/Baya Weaver (Ploceus philippinus), Dhooli Bawk, Sarosh/Eastern Great Egret (Ardea modesta), Kaali Bawk, Machranga/Kingfisher (Halcyn smyrensis), Eagle, Koyal/Kokil (Eudynamys scolopaceus), Baali, Dhanesh/Indian grey hornbill(Ocyceros birostris), Baijja hash, blue throated barbet (Megalaima asiatica), Duck (Anatidae anatinae), Dhar Bawk/Egret, Konch Bawk/Pond Heron (Ardeola grayii) are found in the study area. Photographs of the birds observed in the study area are given in Figure-4. Some aquatic birds like Northern Pintail, etc. were also seen during visit.

Figure-4: Photographs Showing Birds in the Study Area

Butterfly

Presence of butterfly was found in the settlement areas, fields, road side plantation and at bank of river. Some of the common butterfly of the Chattogram region are Gaudy Baron, Golden Browning, Leopard Lace, Clipper, Common Batwing, Common Gem, Orchid, Yellow Hellen etc.

RET Species

As per ecological assessment, no rare, endangered or threatened species are present at the site and in buffer area. None of the species recorded are listed in Red book of IUCN.

### Aquatic Ecology in Buffer Zone

Feni River

Fisheries of Feni River and Estuary

Consultation was carried with the fishermen in study area to gain knowledge about the fish species in canals, River Feni, Khals, Beels, ponds etc. Fish species in Feni River Estuary system is discussed and detailed in Fisheries section given above in this chapter. Fish species occurring in Canals & ponds are: Golda chingri, Bagda chingri, Chiring, Pangash fish, Coral fish, Promphet fish, Catla Catla, Ruhi, Hilsa, Bata fish, Gulla, Pua, Riksha, Lorka, Senuwa, Loitta, Nylotika, Mud crabs, Holona, mrigal, silv er carp, gras carp, karpio, barbs (putis), Chitol, Folai, catfish (Tengra, Singi, Magur, Boal, Pungus), Snakehead (Shol, Taki), bele etc.

As per a joint study carried out by group of experts from Bangladesh and India (Ahsan et al., 2014) as mentioned in Fifth National Report on Bangladesh to the Convention on Biological Diversity, Estuary area of Feni River in Mirsarai is one of potential spawning zone of Hilsa (Tenualosa islisha) fish. Spawning season of Hilsa fish is September to October. However, the peak time of spawning is for 11 days (15-24 October). Thus any activity having potential to affect aquatic fauna like dredging of sand should not be undertaken during this period as it may affect spawning activity of Hilsa.

Planktons of Feni River and Estuary

Planktons which occurs in waters of River Feni are of following genus: Spirogyra, Zygnema, Volvox, Pediastrum, Anabaena, Desmidium, Gonatozygon, Mougeotia and Microcystis and the zooplankton genera were, Asplanchna, Notholca, Daphnia, Filinia and Cyclops. Crustacean larvae (Nauplii) are also found in river water

Bay of Bengal (Sea)

A study carried out by NGO YPSA (Shipbreaking Activities in Bangladesh and collision of Marine Biodiversityby Prabal Barua) is available for Sandwip which is app. 18 from EZ (Zone 2A & 2B) site. As per the study the ecological diversity in this section of sea is given below

Planktons

Major phytoplanktons in these waters are Aanabena, Clostratrum, Coscinodiscus, Euglena&Zygnema. Out of these Aanabena (30.41%) dominates during monsoon and Coscinodiscus (97.5%) dominates during post monsoon. Abundance of the phytoplanktons in the area is 411 cells/l in monsoon season and 190 cells / l in post monsoon season. Major zoo-planktons in these waters areCalanoida, Cyclopedia, Sagitta, Lucifer, Zoea, Acetes shrimp & Cladocera. Out of these Calanoida, Cylpclpedia, Sagitta and Zoea dominates during monsoon and Calanoida, Acetes shrimp, Lucifer, Cladocera and Zoea during post monsoon.

Among the macro benthos Amphipod, Polychaete, Nemertina, Fish egg, Cladocera, Calanoida and Polychaete are found. Out of these presence of Amphipod, Polychaete, Nemertina, Fish egg dominates during the monsoon and Cladocera, Nemertina, Calanoida and Polychaetedominates in the post monsoon season. Abundance of the macro benthos can be thus said to be 368.28 ind./ m3 in monsoon season and 14204.41 ind./m3 in post monsoon season.

Fisheries

Some of the commercially important fishes of these waters majorly include Indian salmon (Polynemus indicus) commonly known as Lakhua; grouper (Epinephelus lanceolatus) known as bole coral; Long jew fish (Otolithoides brunneus) locally known as lombu fish; spanish mackerel (Cybium guttatum) known as maitta; butter fish (Psenes indicus); River shad (Tenualusa ilisha); Jwelled shad (Ilisha filigera) locally known as choikka; mango fish (Polynemus paradysius) known as ‘Hriska Machh’; silver pomfret (Stromateus chinensis); bombay duck (Herpodon nehereus); mullet (Mugil cephalus); Sea bass/Bhetki (Lates calcarifer) and Anchovy(Coilia dussumeri; Coilia ramkorati; Setipinna taty); mud skipper/Baila (Gobius melanosoma, G. Sadanundio); Koiputi match (Anodontostoma chacunda); Kata mach (Arius thalassinus), Lahmuri mach (Carangoides malbaricus); Dora mach (Apocryptes serperaster) and Lakkha (Polynemus indicus). Major marine fishing grounds and marine reserves in the Bay of Bengal are shown in Figure 1.4-5.

Source: Ecosystem Health and Management of Pollution in the Bay of Bengal (BFRI)

Figure 1.4-5: Fishing Grounds and marine reserves in Bay of Bengal

Coral Reefs

Coral reefs are not expected to occur in this region of sea. St. Martin’s Island is the only coral reef island in Bangladesh, locally known as Jinjiradwip (Institute of Marine Sciences, University of Chattogram, Bangladesh). Little information is available on Bangladesh’s Off-shore coral presence. Off-shore corals are reported to be present around St. Martin’s Island area only.

APPENDIX I: SAMPLE GRIEVANCE REGISTRATION FORM

| Grievance Form : Bangladesh Economic Zones Authority |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Grievance reference number (to be completed by Project): |  |  |  |  |  |  |  |
| Contact details (may be submitted anonymously) | Name (s): |  |  |  |  |  |  |
|  | Address: |  |  |  |  |  |  |
|  | Telephone: |  |  |  |  |  |  |
|  | Email: |  |  |  |  |  |  |
| How would you prefer to be contacted (check one) | By mail/post: ☐ |  |  | By phone: ☐ |  |  | By email ☐ |
| Preferred language | ☐ Bangla |  |  | ☐ English |  |  |  |
| Provide details of your grievance. Please describe the problem, who it happened to, when and where it happened, how many times, etc. Describe in as much detail as possible. |  |  |  |  |  |  |  |
| What is your suggested resolution for the grievance, if you have one? Is there something you would like BEZA or another party/person to do to solve the problem? |  |  |  |  |  |  |  |
| How have you submitted this form to the project? | Website ☐ |  |  | Email ☐ |  |  | By hand ☐ |
|  | In person ☐ |  |  | By telephone ☐ |  |  | Other (specify) ☐ |
| Who filled out this form (If not the person named above)? | Name and contact details: |  |  |  |  |  |  |
| Signature |  |  |  |  |  |  |  |
| Name of BEZA official assigned responsibility |  |  |  |  |  |  |  |
| Resolved or referred to GRC1? | ☐ Resolved |  | ☐ Referred |  | If referred, date: |  |  |
| Resolved referred to GRC2? | ☐ Resolved |  | ☐ Referred |  | If referred, date: |  |  |
| Completion |  |  |  |  |  |  |  |
| Final resolution (briefly describe) |  |  |  |  |  |  |  |
|  | Short description |  |  |  |  | Accepted? (Y/N) | Acknowledgement signature |
| 1st proposed solution |  |  |  |  |  |  |  |
| 2nd proposed solution |  |  |  |  |  |  |  |
| 3rd proposed solution |  |  |  |  |  |  |  |

APPENDIX J: GREEN BELT DEVELOPMENT PLAN

## Introduction

No tree cutting is proposed to be undertaken as per the current proposal of the EZ development. It is proposed to develop green buffer along the periphery and along the canal. The plantation in the green buffer should be carried out in accordance to the following guidelines.

## Pattern of the Green Belt

A green belt of width 30 m is suggested to be developed all along the boundary of EZ and along Ichakhali channel. 6 rows of plantation can be planted in this buffer. Native plantation of the region should be planted. The green belt should consist of herbs, shrubs, short-medium height trees and tall trees. First two rows should be planted with herbs and shrubs. Next two rows should be short to medium heighted trees and last two rows should be planted with tall trees. Space of 1X1 m should be kept for planting herbs, space of 3X 3 m should be kept for planting shrubs and small heighted trees and space of 5X 5 m should be kept for planting medium and tall trees. The pits are then filled with earth, sand, silt and manure in pre-determined proportions. Saplings planted in such pits are watered liberally during dry months. The pits are to be filled with earth, sand, silt and manure in pre-determined proportions. Saplings planted in such pits are watered liberally during dry months.

## Selection of Tree Species

The Project involve movement of vehicle for transportation of material Thus emissions like particulate matter, SO2, NOx & CO shall be generated at site. Also there is potential of generation of coal dust while unloading the materials at stock piles. Thus the plantation species tolerant to these pollutants and mitigate these from air shall be planted. Species selecting criteria is given below:

    - Native species including Mangroves

    - Tolerant to expected pollutants at site

    - Longer duration of foliage

    - Freely exposed foliage (adequate height of crown, openness of foliage, big leaves, small stomata apertures, stomata well exposed)

    - Leaves supported on firm petioles

## Recommended Plant species

Based on nature of pollutants following tree species are recommended to be planted

| Species Name | Local Name | Family |
| --- | --- | --- |
| Acacia moniliformes | Akashmoni | Leguminosae |
| Areca catechu | Supari | Palmae |
| Avecenia alba/ Avicennia Marinavierh | Baen | Aviceniaceae |
| Azadirachta indica | Neem | Meliaceae |
| Carica papaya | Pepey | Caricaceae |
| Cocos nucifera | Narikel | Palmae |
| Excoecaria agallocha | Gewa | Euphorbiaceae |
| Herritiera fomes | Sundari | Sterculiaceae |
| Mangifera indica | Aam | Anacardiaceae |
| Manilkara zapota | Safeda | Zapotaceae |
| Phoenix sylvestris | Khejur | Palmae |
| Psidium guajava | Peyara | Myrtaceae |
| Sonneratia apetala | Kewda/Keora | Lyrthaceae |
| Spondius pinnata | Amra | Anacardiaceae |
| Swietenia mehagoni | Mehogani | Meliaceae |
| Syzygium cumini | Jaam | Myrtaceae |
| Syzygium samarengense | Jamrul | Myrtaceae |
| Tamarindus indica | Tentul | Leguminosae |
| Terminalia catapa | Kathbadam | Combrataceae |
| Tectonia grandis | Teak | Lamiaceae |
| Gmelina arborea | Gamari | Verbenaceae |
| Brassica juncea | Raisharisha | Brassicaceae |
| Olea europaea | Olives | Oleaceae |
| Terminalia bellirica | Bohera | Combretaceae |
| Terminalia chebula | Horitoki | Combretaceae |
| Phyllanthus emblica | Amla | Phyllanthaceae |
| Protium serratum | Gutgutiya | Burseraceae |
| Propospis cineraria | Jand | Fabaceae |
| Casuarina equisetifolia | Jhao Ghas | Casuarinaceae |
| Albizia lebbeck | Koroi | Fabaceae |
| Delonix regia | Krishnachura | Fabaceae |
| Leucaena leucocephala | Ipil Ipil | Fabaceae |
| Carissa carandas | Karamcha tree | Apocynaceae |
| Artocarpus heterophyllus | Jack fruit | Moraceae |
| Polyalthia longifolia | Pseudo Ashoka | Annonaceae |
| Musa parasisiaca | Banana | Musaceae |
| Garuga pinnat | Bhadi | Burseraceae |
| Abelmoschus esculentu | Bhendi | Malvaceae |
| Erythrina indica | Mandar | Fabaceae |
| Acacia mangium | Mangiam | Fabaceae |
| Dalbergia sissoo | Sheeshu | Fabaceae |
| Lawsonia inermis | Kat Mehndi | Lythraceae |
| Swietenia mahagony | Mahagony | Meliaceae |
| Arundinaria | Cane Ghas | Poaceae |
| Samanea saman | Rain Tree | Fabaceae |
| Zizyphus mauritiana | Boroi | Rhamnaceae |
| Calotropis | Calotropis | Apocynaceae |
| Ceriops decandra | Gora | Rhizophoraceae |
| Litchi chinensis | Litchi | Sapindaceae |
| Ficus benjamina | Dumur | Moraceae |
| Citrus maxima/Citrus grandis | Jambura | Rutaceae |
| Mimosa pudica | Lazzabati | Fabaceae |
| Pinus densiflora | Pine | Pinaceae |
| Excoecaria agallocha | Gewa | Euphorbiaceae |
| Nypa fruticans | Golpata | Arecaceae |
| Heritiera littoralis | Sundri | Lauraceae |

## Plantation Methodology

Components of green belts on roadside fence should be both absorbers of gases as well as of dust particles, including even lead particulates. Thus the choice of plants should include pollution tolerant shrubs of height 1 to 1.5 m and trees of 3 to 5m. The intermixing of trees and shrubs should be such that the foliage area density in vertical is almost uniform. For effective removal of pollutants, it is necessary that (i) plants should grow under conditions of adequate nutrient supply, (ii) absence of water stress and (iii) plants are well exposed to atmospheric conditions (light & breeze).

Multiple rows of green belt shall be developed. Green belt should be pyramidal in shape. Plantation pattern shall be kept as given below:

- Short trees and tall shrubs shall be planted as first row (from road) followed by tall tree plantation which will be followed by another row of medium and small trees and tall shrubs.

- Planting of trees should be in appropriate encircling rows, each rows alternating the previous one to prevent further fanning and horizontal pollution dispersion;

- Since tree trunks are normally devoid of foliage, it would be appropriate to have small shrubs in front and in between the tree spaces;

- The open areas between the process installations where trees cannot be planted should be covered with lawn grasses for effective trapping and absorptions of air pollutants.

- Fast growing trees with thick canopy and perennial foliage should be selected so that the effective tree height with envisaged objective will be attained in minimum span of time

## Time of Plantation

Plantation of the tree sapling should be done only after the first shower during the rainy season. The best time for plantation is after 15 days from the day of first shower during rainy season.

## Protection of Tree saplings

Circular tree guard should be placed after the plantation of the saplings for the protection of these young plants from the ravages of cattle, sheep and goat and other animals. If tree saplings died or damage occur after placing the circular tree guard, timely replacements of damaged plant and thereafter care is important.

## After Care & Monitoring

The growing plants are cared at least for the first two years under favourable conditions of climate and irrigation. Nutrients in pits are supplemented and the juveniles provided protection.

Thinning shall start after the stand is 3-4 years old and repeated every 4 years until the stand is 15 years old. Between 15-25 years old, thinning should be conducted every 5 years and after 25 years old, thinning shall be done after every 10 years. When the canopy closes, at about 6 years, 30-40% of the stems shall be thinned to selectively remove suppressed, diseased and badly formed trees.

Periodic assessment shall be carried for survivability of the trees. Minimum 70% survival rate shall be achieved.

## Records Keeping & Reporting

The following records shall be maintained:

- Record of Tree plantation

- Record of Survivability rate

Inspection shall be carried out at site to know the survival rate of the plantation. The tree plantation and survivability report shall be prepared every six monthly.

## Responsibility

Compensatory plantation shall be carried out by forest department. Survival rate of plantation shall be inspected of the by BEZA. Plantation within the terminal/jetty/lock site shall be carried out by BEZA and shall be monitored by BEZA.

APPENDIX K: TEMPORARY DRAINAGE MANAGEMENT PLAN

| Project activity/ Impact source | Environmental Impact | Mitigation Measures/ Management Guidelines |
| --- | --- | --- |
| Excavation and earthworks and construction yards | Lack of proper drainage for rainwater/liquid waste or wastewater owing to the construction activities harms environment in terms of water and soil contamination and mosquito growth. | The Contractor shall Prepare a program for prevent/avoid standing waters, which PMU will verify in advance and confirm during implementation. Provide alternative drainage for rainwater if the construction works/earth-fillings cut the established drainage line. Establish local drainage line with appropriate silt collector and silt screen for rainwater or wastewater connecting to the existing established drainage lines already there Rehabilitate road drainage structures immediately if damaged by contractors’ road transports. Build new drainage lines as appropriate and required for wastewater from construction yards connecting to the available nearby recipient water bodies. Ensure wastewater quality conforms to the relevant standards provided by DoE, before it being discharged into the recipient water bodies. Ensure the internal roads/hard surfaces in the construction yards/construction camps that generate storm water drainage to accommodate high runoff during downpour and that there is no stagnant water in the area at the end of the downpour. Construct wide drains instead of deep drains to avoid sand deposition in the drains that require frequent cleaning. Provide appropriate silt collector and silt screen at the inlet and manholes and periodically clean the drainage system to avoid drainage congestion. Protect natural slopes of drainage channels to ensure adequate storm water drains. Regularly inspect and maintain all drainage channels to assess and alleviate any drainage congestion problem. Reduce infiltration of contaminated drainage through storm water management design. |
| Ponding of water | Health hazards due to mosquito breeding. | - Do not allow ponding of water especially near the waste storage areas and construction and labor camps. - Discard all the storage containers that are capable of storing of water, after use or store them in inverted position. |

# Appendices L: Photographs of Stakeholder consultation meetings and FGDs

| KII with UNO, Mirsarai Upazila, 6-11-2019 | KII with SUFO, Mirsarai Upazila, 6-11-2019 |
| --- | --- |
| KII with Forest Range Officer of Mirsarai Upazila, 6-11-2019 | KII with ULO, Mirsarai Upazila, 6-11-2019 |
| KII with Upazila Agriculture Officer, Mirsarai, 6-11-2019 | KII with Social Welfare Officer, Mirsarai, 6-11-2019 |
| KII with Upazila Engineer, Mirsarai, 6-11-2019 |  |

Figure 1: Key Informants Interview (KII) with Upazila Officers, Mirsarai

| Focus Group Discussion with local stakeholders at Ichakhali Switch Gate, 9no Ward of 6no Ichakhali Union, 06-11-2019 | Public consultation with local stakeholders at Jolodas Para , Dhomkhali, 8no Ward, 16no Shaherkhali Union, 07-11-19 |
| --- | --- |
| Focus Group Discussion with local stakeholders at Bangla Bazar, 9 no Ward of 6no Ichakhali Union, 07-11-19 | Group Discussion with local stakeholders at Abu Torab Bazar of 11no Moghadia Union, 05-11-19 |
| Public consultation Discussion with local stakeholders at Chorshorot, 9no Ward of 06 Ichakhali Union, 07-11-19 | Focus Group Discussion with local stakeholders at Beribadh Chorshorot, 9no Ward of 6 Ichakhali Union, 07-11-19 |
| Focus Group Discussion with local stakeholders at Moghadia Absar Chouwdury High School of 11no Moghadia Union, 05-11-19 | Walk in interview with local stakeholders at 5no Ward of 11no Moghadia Union, 07-11-19 |
| Group Discussion with local stakeholders at Chorshorot, 9no Ward of 06 Ichakhali Union, 07-11-19 | Group Discussion with local stakeholders at Moghadia Anjuman Nesa Govt. Primary School of 11no Moghadia Union, 05-11-19 |

Figure 2: Public consultations, FGDs, group discussions & walk in interviews with the stakeholders

# Appendices M: List of Industries in Zone 2A and 2B

| SL. No. | Name of Industry | Type of Industry | Allotted Plot Size |
| --- | --- | --- | --- |
| ZONE 2A |  |  |  |  |
| 1 | ACI Ltd | Food Processing | 99.9 |
| 2 | Administration | Administration | 3.6 |
| 3 | Alif Embroidery Village Ltd | Textile & Garments | 10.1 |
| 4 | Allotted | Allotted Plots | 37.1 |
| 5 | Aman Spining Mills Ltd. | Textile & Garments | 30.2 |
| 6 | Arab Bangladesh foods Ltd. | Food Processing | 10 |
| 7 | Aust-Bangla (JV) Accessories Industries Ltd | Other | 10.1 |
| 8 | BDCom Online Ltd. | Electronics | 10 |
| 9 | BEZA | BEZA | 0.8 |
| 10 | BSA Fashion Ltd | Textile & Garments | 24.8 |
| 11 | BTCL | Utilities | 0.6 |
| 12 | CETP | CETP | 20.9 |
| 13 | Commercial | Commercial | 0.6 |
| 14 | Energy & Power | Energy & Power | 2.3 |
| 15 | Energypac Power Generation Company Ltd. | Energy & Power | 29.1 |
| 16 | Eonmetall International Ltd | Steel & Steel Product | 10 |
| 17 | Eurasia Food Precessing Ltd | Food Processing | 10 |
| 18 | Fakhruddin Textile | Textile & Garments | 50.2 |
| 19 | Fawn International | Energy & Power | 25.1 |
| 20 | Gas-1 Ltd | Energy & Power | 25 |
| 21 | Green | Green | 1.3 |
| 22 | Green Health Ltd | Pharmaceutical | 10 |
| 23 | Hamko Corporation Ltd | Battery | 10 |
| 24 | Integra Apparels (Bangladesh) Ltd. | Textile & Garments | 10.2 |
| 25 | Jahangir Stainless Steel Ltd. | Steel & Steel Product | 10 |
| 26 | Jamuna Spacetech Jv. Ltd | Energy & Power | 40 |
| 27 | Juhana Textile Ltd. | Textile & Garments | 10 |
| 28 | Maheen Dizayn Etiket (BD) Ltd. | Textile & Garments | 10 |
| 29 | Merchant Melbourne Pte Ltd | Food Processing | 9.9 |
| 30 | Nafa Apparels Ltd+Aswad Composite Mills Ltd. | Textile & Garments | 70.2 |
| 31 | Overseas Marketing Corporation (Pvt) Ltd | Textile & Garments | 19.6 |
| 32 | Quality Fashion Wear Ltd | Textile & Garments | 10 |
| 33 | Raquef Apparels Washing and Packaging Industry | Textile & Garments | 30.3 |
| 34 | Ratools Apparels Ltd | Textile & Garments | 10 |
| 35 | REB | Utilities | 2.5 |
| 36 | Religious | Religious | 0.9 |
| 37 | Reserved for BEZA | BEZA | 10 |
| 38 | Reservoir | Reservoir | 0.5 |
| 39 | Reza Fashion Ltd | Textile & Garments | 10 |
| 40 | Road | Road | 94.9 |
| 41 | Security | Security | 5 |
| 42 | Surjin Tech Co. Ltd | Utilities | 1.9 |
| 43 | Textile & Garments | Textile & Garments | 5 |
| 44 | Unallotted | Unallotted | 117.2 |
| 45 | Utilities | Utilities | 5 |
| 46 | Well Composite Knite Ltd.(Sanji Textile) | Textile & Garments | 20 |
| ZONE 2B |  |  |  |  |
| 1 | Adila Apparels | Textile & Garments | 1 |
| 2 | Advance World Limited | Textile & Garments | 1.1 |
| 3 | Afrah Dresses Ltd. | Textile & Garments | 2 |
| 4 | Al-Ittefaq Textile Ltd. | Textile & Garments | 1 |
| 5 | Alps Apparels Ltd. | Textile & Garments | 3 |
| 6 | Apparel Gallery Limited | Textile & Garments | 33 |
| 7 | ARL Dresses Limited | Textile & Garments | 2 |
| 8 | Artistic Design Limited | Textile & Garments | 34 |
| 9 | Avant Garments Limited | Textile & Garments | 5 |
| 10 | Babylon Casualwear Limited | Textile & Garments | 5 |
| 11 | Babylon Garments Limited | Textile & Garments | 5 |
| 12 | BEZA | BEZA | 21.6 |
| 13 | BLP Warm Fashion Ltd. | Textile & Garments | 1 |
| 14 | CETP | CETP | 5 |
| 15 | Charm Fashion | Textile & Garments | 5 |
| 16 | Chowdhury Fashion Wear Limited | Textile & Garments | 2 |
| 17 | Clifton Apparels Limited | Textile & Garments | 5 |
| 18 | Clifton Cotton Mills Limited | Textile & Garments | 5 |
| 19 | Columbia Apparels Ltd. | Textile & Garments | 6.1 |
| 20 | Columbia Garments Ltd. | Textile & Garments | 6.1 |
| 21 | Cotton Dyeing & Finishing Mills Ltd. | Textile & Garments | 2 |
| 22 | Cotton Line (BD) Ltd. | Textile & Garments | 2 |
| 23 | Denim Fashions Ltd. | Textile & Garments | 2 |
| 24 | EH Fabrics Ltd. | Textile & Garments | 2 |
| 25 | Energypac Fashions Ltd. | Textile & Garments | 3 |
| 26 | Epyllion Style Limited | Textile & Garments | 20 |
| 27 | Eva Sweater Ltd. | Textile & Garments | 3 |
| 28 | Farseeing Knit Composite Ltd. | Textile & Garments | 2 |
| 29 | Fountain Garments Manufacturing Ltd. | Textile & Garments | 3 |
| 30 | Four H Fashion Limited | Textile & Garments | 5 |
| 31 | Garments Home (Pvt.) Ltd. | Textile & Garments | 1 |
| 32 | Genesis Fashions Ltd. | Textile & Garments | 8 |
| 33 | Global Shirt Limited | Textile & Garments | 20.1 |
| 34 | Graphics Textile Limited | Textile & Garments | 10 |
| 35 | Green | Green | 18.6 |
| 36 | Image Garments Ltd. | Textile & Garments | 2 |
| 37 | Impressive Knit Composite | Textile & Garments | 2 |
| 38 | Integra Apparel (BD) Limited | Textile & Garments | 10.1 |
| 39 | Karim Textiles Limited | Textile & Garments | 5 |
| 40 | Kay Garments Limited | Textile & Garments | 1 |
| 41 | M/S ARL Apparels Limited | Textile & Garments | 3 |
| 42 | Madina Garments Limited | Textile & Garments | 1 |
| 43 | Majumder Garments Limited | Textile & Garments | 5.1 |
| 44 | Mansheen Co (BD) Ltd. | Textile & Garments | 1 |
| 45 | Mars Sportswear Limited | Textile & Garments | 10.1 |
| 46 | Mellow Fashion Limited | Textile & Garments | 1 |
| 47 | Mens Fashion Limited | Textile & Garments | 20 |
| 48 | Musk Trousers Ltd. | Textile & Garments | 1 |
| 49 | Pacific Cotton Ltd. | Textile & Garments | 2 |
| 50 | Posh Garments Ltd. | Textile & Garments | 3 |
| 51 | Progressive Apparels Ind. Ltd | Textile & Garments | 1 |
| 52 | R.O Textile Mills (Pvt) Ltd. | Textile & Garments | 3 |
| 53 | RDM Apparels Ltd. | Textile & Garments | 6.1 |
| 54 | RNSCO Sweaters Limited | Textile & Garments | 1 |
| 55 | Road | Road | 53.5 |
| 56 | RR Fashion Ltd. | Textile & Garments | 1 |
| 57 | SAMS Attire Ltd. | Textile & Garments | 2 |
| 58 | SF Denim Apparels Limited | Textile & Garments | 5.1 |
| 59 | Shamser Apparels Ltd. | Textile & Garments | 1 |
| 60 | Shamser Rezia Fashion Ltd. | Textile & Garments | 1.1 |
| 61 | SQ Celsious Limited | Textile & Garments | 10 |
| 62 | S-six Fashions (Pvt.) Ltd | Textile & Garments | 1 |
| 63 | Sterling Denims Limited | Textile & Garments | 30 |
| 64 | Tafof Apparels Ltd. | Textile & Garments | 2 |
| 65 | TBH Fashion Tex (Pvt.) Ltd. | Textile & Garments | 1 |
| 66 | That's it Sports Wear Ltd | Textile & Garments | 33 |
| 67 | Top Star Fashions Limited | Textile & Garments | 1 |
| 68 | Towel Tex Ltd. | Textile & Garments | 1 |
| 69 | Tusuka Jeans Ltd. | Textile & Garments | 2 |
| 70 | Tusuka Trousers Limited | Textile & Garments | 4.1 |
| 71 | Unigears Limited | Textile & Garments | 15 |
| 72 | Uranus Apparels Ltd. | Textile & Garments | 2 |
| 73 | Vancot Limited | Textile & Garments | 5 |
| 74 | Visual Knitwears Limited | Textile & Garments | 3.9 |
| 75 | W. Apparels | Textile & Garments | 6 |
| 76 | Well Fashion Limited | Textile & Garments | 2 |